# 太空垃圾高速捕获机械臂仿真系统——基于视觉预测的轨迹规划研究

# Abstract

This paper presents an engineering-oriented simulation workflow for space-debris capture and reports reproducible quantitative results from the latest script version.

The experiment uses batch execution with fixed seeds (6 episodes, 600 s simulated time per episode, 100x speedup), and records both CSV metrics and replayable videos.

From the 2026-03-03 run, the system achieves a destroy rate of 0.000, mean captures of 2.167 (range 1-4), and mean delta_v_total of 9945.333 (std 1004.144). Additional derived metrics (capture efficiency and capture frequency) are reported for cross-episode comparison.

This revision focuses on reproducibility, metric consistency, and evidence traceability (commands -> data -> videos), providing a practical baseline for larger-scale ablation and policy comparison in future work.

# Keywords: space debris, robotic capture, simulation, reproducibility, quantitative evaluation, trajectory planning

# 1 引言

## 1.1 研究背景

截至目前，地球轨道上已有超过36,000个大于10cm的空间碎片被跟踪编目，而小于1cm的碎片数量估计超过1.3亿个。这些高速运动的空间碎片对在轨航天器构成严重威胁——即使是毫米级碎片，在轨道速度（约7.8km/s）下也具有巨大的动能。空间碎片主动清除（Active Debris Removal, ADR）已成为国际航天界的重要研究方向。

欧洲航天局（ESA）的ClearSpace-1任务计划于2026年发射，旨在验证空间碎片捕获技术。日本宇宙航空研究开发机构（JAXA）的ELSA-d实验已于2021年成功演示了磁性对接捕获技术。这些任务表明，机械臂抓取是空间碎片清除的核心技术路径之一。

## 1.2 技术挑战

太空垃圾捕获面临以下关键技术挑战：

（1）高速相对运动：即使在交会接近后，目标碎片仍存在显著的相对运动（可达数m/s），对机械臂的响应速度提出极高要求；（2）目标非合作性：碎片通常处于无控翻滚状态，三轴旋转叠加使得姿态变化复杂且难以预测；（3）形状不规则性：废弃卫星、火箭残骸、碎片云等目标形状各异，抓取点选择困难；（4）微重力环境：微重力条件下的动力学特性与地面环境显著不同；（5）通信延迟：地面遥操作存在显著时延，要求系统具备自主决策能力。

## 1.3 研究创新点

本研究的核心创新在于：采用深度学习（LSTM网络）进行目标运动预测，结合模型预测控制（MPC）实现"预判式"抓取轨迹规划。与传统的反应式控制策略不同，本方法通过预测目标未来0.5-2秒的运动轨迹，提前规划机械臂的拦截路径，从而有效应对高速运动目标的捕获挑战。

## 1.4 论文结构

本文第2节介绍系统总体架构设计；第3节详细阐述各核心算法；第4节描述仿真实验设计与实施；第5节分析实验结果；第6节总结全文并展望未来工作。

# 2 系统总体架构

## 2.1 系统组成

本仿真系统由六个核心模块组成，如图1所示：

┌─────────────────────────────────────────────────────────────┐
│                    太空环境仿真模块                           │
│  ┌──────────┐  ┌──────────┐  ┌──────────┐                  │
│  │ 碎片运动  │  │ 微重力   │  │ 多形状   │                  │
│  │ 动力学    │  │ 环境     │  │ 碎片模型  │                  │
│  └────┬─────┘  └──────────┘  └──────────┘                  │
│       │                                                      │
│  ┌────▼─────────────────┐                                   │
│  │   视觉感知模块        │                                   │
│  │  (模拟传感器+噪声)    │                                   │
│  └────┬─────────────────┘                                   │
│       │ 观测序列                                             │
│  ┌────▼─────────────────┐    ┌──────────────────┐          │
│  │   运动预测模块        │───▶│  轨迹规划模块     │          │
│  │  (LSTM / 物理基线)    │    │  (MPC优化)        │          │
│  └──────────────────────┘    └────┬─────────────┘          │
│                                    │ 目标轨迹                │
│  ┌─────────────────────────────────▼────────────┐          │
│  │          抓取执行模块                          │          │
│  │  ┌──────────┐  ┌──────────┐  ┌──────────┐   │          │
│  │  │ 6-DOF    │  │ 逆运动学  │  │ 末端执行器│   │          │
│  │  │ 机械臂   │  │ (DLS-IK) │  │ 策略选择  │   │          │
│  │  └──────────┘  └──────────┘  └──────────┘   │          │
│  └──────────────────────────────────────────────┘          │
│                                                              │
│  ┌──────────────────────────────────────────────┐          │
│  │          性能评估模块                          │          │
│  │  成功率 | 响应时间 | 相对速度 | 预测误差       │          │
│  └──────────────────────────────────────────────┘          │
└─────────────────────────────────────────────────────────────┘

## 2.2 变量设计

自变量（实验控制参数）：
- 目标线速度：0.1 ~ 6.0 m/s
- 目标旋转角速度：0 ~ 60°/s（0 ~ 1.05 rad/s）
- 目标形状：废弃卫星、火箭残骸、不规则碎片

因变量（性能指标）：
- 抓取成功率
- 抓取响应时间（从检测到抓取成功的时间）
- 末端执行器与目标的相对速度差
- 最小接近距离

控制变量：
- 机械臂自由度：6-DOF
- 关节最大角速度：3.14 rad/s
- 视觉系统帧率：30 Hz
- 传感器噪声标准差：0.01 m

# 3 核心算法

## 3.1 太空碎片运动模型

### 3.1.1 平移运动

在微重力环境下，碎片不受外力作用，做匀速直线运动：

𝐩(t+Δt)=𝐩(t)+𝐯·Δt

其中 𝐩∈ℝ3 为位置向量，𝐯∈ℝ3 为速度向量。

### 3.1.2 旋转运动

碎片的旋转运动由欧拉方程描述。对于刚体在无外力矩条件下：

𝐈ω˙+ω×(𝐈ω)=0

其中 𝐈 为惯性张量矩阵，ω 为角速度向量。

惯性张量按长方体近似计算：

𝐈=diagm(sy2+sz2)12m(sx2+sz2)12m(sx2+sy2)12

角速度更新：

α=𝐈−1(−ω×𝐈ω)$$$$ω(t+Δt)=ω(t)+α·Δt

注意：对于非对称惯性张量（Ix≠Iy≠Iz），即使无外力矩，角速度方向也会随时间变化，这正是太空碎片复杂翻滚运动的物理根源。

### 3.1.3 姿态表示与更新

采用四元数 𝐪=(qw,qx,qy,qz) 表示姿态，避免万向锁问题。四元数积分更新：

Δ𝐪=cosθ2 ω^sinθ2

其中 θ=|ω|Δt，ω^=ω/|ω|。

𝐪(t+Δt)=𝐪(t)⊗Δ𝐪

四元数乘法定义为：

𝐪1⊗𝐪2=(w1w2−x1x2−y1y2−z1z2 w1x2+x1w2+y1z2−z1y2 w1y2−x1z2+y1w2+z1x2 w1z2+x1y2−y1x2+z1w2)

## 3.2 6-DOF机械臂运动学

### 3.2.1 DH参数与正运动学

采用标准Denavit-Hartenberg（DH）参数描述机械臂构型。每个关节的齐次变换矩阵为：

𝐓i=(cθi−sθicαisθisαiaicθi sθicθicαi−cθisαiaisθi 0sαicαidi 0001)

其中 ai,αi,di,θi 分别为连杆长度、连杆扭转角、连杆偏距和关节角。

末端执行器位姿通过连乘得到：

$$\mathbf{T}{0}^{6} = \prod$$}^{6} \mathbf{T}_{i-1}^{i

本系统机械臂DH参数如表1所示：

| 关节 | ai (m) | αi (rad) | di (m) | 功能 |
| --- | --- | --- | --- | --- |
| 1 | 0 | π/2 | 1.0 | 基座旋转 |
| 2 | 1.5 | 0 | 0 | 肩部俯仰 |
| 3 | 1.2 | 0 | 0 | 肘部弯曲 |
| 4 | 0 | π/2 | 0 | 腕部旋转 |
| 5 | 0 | -π/2 | 0.8 | 腕部俯仰 |
| 6 | 0 | 0 | 0.8 | 末端旋转 |

总臂展：5.3m

### 3.2.2 雅可比矩阵

雅可比矩阵 𝐉∈ℝ3×6 建立末端速度与关节速度的映射关系：

𝐱˙=𝐉(𝐪)𝐪˙

本系统采用数值微分法计算雅可比矩阵：

Jij=fi(𝐪+ϵ𝐞j)−fi(𝐪)ϵ

其中 ϵ=10−6，𝐞j 为第 j 个关节方向的单位向量，fi 为正运动学函数的第 i 个分量。

### 3.2.3 阻尼最小二乘逆运动学（DLS-IK）

传统雅可比伪逆法在奇异位形附近会产生极大的关节速度。本系统采用阻尼最小二乘（Damped Least Squares, DLS）方法：

Δ𝐪=𝐉T(𝐉𝐉T+λ2𝐈)−1Δ𝐱

其中 λ=0.01 为阻尼系数，$\Delta\mathbf{x} = \mathbf{x}{target} - \mathbf{x}$ 为末端位置误差。

关节速度限制：

$$\Delta\mathbf{q}{limited} = \begin{cases} \Delta\mathbf{q} & \text{if } \max|\Delta q_i| \leq \dot{q}$$}\Delta t \ \Delta\mathbf{q} \cdot \frac{\dot{q}_{max}\Delta t}{\max|\Delta q_i|} & \text{otherwise} \end{cases

### 3.2.4 可操作度指标

采用Yoshikawa可操作度指标评估机械臂的运动灵活性：

w=det(𝐉𝐉T)

当 w→0 时，机械臂接近奇异位形，运动能力退化。该指标用于MPC代价函数中的奇异位形惩罚项。

## 3.3 视觉感知模块

### 3.3.1 传感器模型

本系统模拟RGB-D深度传感器，以固定帧率采集目标碎片的状态信息。传感器参数如表2所示：

| 参数 | 符号 | 值 | 说明 |
| --- | --- | --- | --- |
| 采集帧率 | fs | 30 Hz | 每秒采集次数 |
| 测量噪声基准 | σ0 | 0.01 m | 近距离噪声标准差 |
| 最大探测距离 | Rmax | 15.0 m | 超出则无法观测 |
| 视场角 | FOV | 90° | 传感器视场范围 |
| 观测缓冲区 | Nbuf | 300帧 | 约10秒历史数据 |

### 3.3.2 距离相关噪声模型

真实传感器的测量精度随距离增大而降低。本系统采用线性距离相关噪声模型：

σ(d)=σ01dRmax

其中 d 为目标距离，σ0 为基准噪声标准差。各状态量的噪声分别为：

（1）位置噪声：𝐧p~𝒩(0,σ(d)2𝐈3)；（2）速度噪声：𝐧v~𝒩(0,(0.5σ(d))2𝐈3)；（3）姿态噪声：𝐧q~𝒩(0,(0.1σ(d))2𝐈4)；（4）角速度噪声：𝐧ω~𝒩(0,(0.3σ(d))2𝐈3)。

观测模型为：

$\hat{\mathbf{p}} = \mathbf{p}{true} + \mathbf{n}_p, \quad \hat{\mathbf{v}} = \mathbf{v}_v$} + \mathbf{n

姿态四元数在加噪后需重新归一化：

$\hat{\mathbf{q}} = \frac{\mathbf{q}{true} + \mathbf{n}_q}{|\mathbf{q}$} + \mathbf{n}_q|

### 3.3.3 观测置信度

传感器输出观测置信度，反映当前测量的可靠程度：

c=clip1−dRmax+ϵ, 0, 1,ϵ~𝒩(0,0.05)

距离越近置信度越高，该值将传递给后续的运动预测和轨迹规划模块。

### 3.3.4 帧率限制与观测缓冲

传感器以固定帧率 fs=30 Hz 工作，仅在采样时刻输出观测数据。两次采样之间系统使用上一帧数据。观测缓冲区保存最近 Nbuf=300 帧数据（约10秒），为LSTM预测模块提供输入序列。

观测序列的提取格式为：

$\mathbf{X} = [\mathbf{x}{t-L+1}, \mathbf{x}$}, \ldots, \mathbf{x}_t] \in \mathbb{R}^{L \times 10

其中 L=30 为序列长度，每帧 𝐱i=[px,py,pz,vx,vy,vz,qw,qx,qy,qz]T。

## 3.4 LSTM运动预测模块

### 3.4.1 问题定义

运动预测的核心任务是：给定目标碎片过去 L 个时刻的观测序列，预测未来 H 个时刻的状态轨迹。形式化表示为：

𝐘^=fLSTM(𝐗),𝐗∈ℝL×10,𝐘^∈ℝH×10

其中 L=30（输入序列长度），H=50（预测时域，对应0.5秒@100Hz）。

### 3.4.2 网络架构

采用两层堆叠LSTM网络，架构如下：

输入层: (batch, 30, 10)
    ↓
LSTM Layer 1: hidden_size=128, dropout=0.1
    ↓
LSTM Layer 2: hidden_size=128
    ↓ (取最后时刻隐状态)
    ├──→ 轨迹预测分支: FC(128→64) → ReLU → FC(64→500) → reshape(50, 10)
    └──→ 置信度分支:   FC(128→32) → ReLU → FC(32→50) → Sigmoid

网络参数统计：

| 层 | 参数量 |
| --- | --- |
| LSTM Layer 1 | 4×(10×128+128×128+128)=71,168 |
| LSTM Layer 2 | 4×(128×128+128×128+128)=131,584 |
| 轨迹预测FC | 128×64+64+64×500+500=40,756 |
| 置信度FC | 128×32+32+32×50+50=5,778 |
| 总计 | 约249K |

### 3.4.3 输入输出格式

输入：经归一化处理的观测序列

𝐱~i=𝐱i−μσ

其中 μ,σ∈ℝ10 为训练集上计算的均值和标准差向量，σ 下界截断为 10−6 以避免除零。

输出：
1. 预测轨迹 𝐘^∈ℝ50×10：经反归一化还原为物理量
2. 置信度序列 𝐜∈[0,1]50：通过Sigmoid激活，表示各预测时刻的可信程度

### 3.4.4 训练数据生成

训练数据通过仿真环境自动生成，覆盖不同运动参数组合：

（1）线速度：v∈[0.1,6.0] m/s，均匀采样；（2）角速度：ω∈[0,1.05] rad/s，均匀采样；（3）碎片形状：卫星、火箭残骸、不规则碎片随机选取；（4）初始方向：随机生成。

每条轨迹仿真30秒，以随机采样方式从轨迹中提取训练样本（输入30帧 + 标签50帧），每条轨迹最多采样20个样本。

### 3.4.5 物理基线预测器

作为对比基线，实现了基于物理模型的线性外推预测器：

𝐩^(t+Δt)=𝐩(t)+𝐯·Δt

其中 𝐯 为最近10帧的平均速度。基线预测器的置信度随时间指数衰减：

c(t)=e−2t

该设计反映了线性外推在长时域预测中精度快速下降的物理事实。

## 3.5 MPC轨迹规划模块

### 3.5.1 规划框架

模型预测控制（MPC）以预测的目标轨迹为参考，在线优化机械臂末端的运动轨迹。MPC参数配置如表3所示：

| 参数 | 符号 | 值 | 说明 |
| --- | --- | --- | --- |
| 预测时域 | Np | 20步 | 前瞻规划步数 |
| 控制时域 | Nc | 10步 | 优化控制步数 |
| 位置跟踪权重 | wpos | 10.0 | 末端位置误差权重 |
| 速度匹配权重 | wvel | 5.0 | 相对速度匹配权重 |
| 力矩平滑权重 | wτ | 0.1 | 关节角度变化量惩罚 |
| 奇异位形惩罚 | wsing | 1.0 | 接近奇异位形的惩罚 |

### 3.5.2 拦截点选择

拦截点选择是MPC规划的关键步骤。系统在预测轨迹上搜索最优拦截点，综合评分函数为：

S(i)=2.0·R(i)+3.0·c(i)+1.0·T(i)

其中各分项定义为：

可达性评分：

$R(i) = \begin{cases} 1 - \frac{|\mathbf{p}i - \mathbf{p}}|}{L_{arm}} & \text{if } |\mathbf{pi - \mathbf{p}$}| \leq 0.95 L_{arm} \ 0 & \text{otherwise} \end{cases

其中 Larm=5.3 m 为机械臂总臂展。

置信度评分：c(i) 直接使用预测模块输出的置信度。

时间评分：

T(i)=e−0.15i

在前10步预测范围内选择 S(i) 最大的点作为拦截目标。

### 3.5.3 平滑轨迹生成

确定拦截点后，采用三次Hermite插值生成平滑的末端轨迹，确保位置和速度的连续性：

$\mathbf{p}(t) = h_{00}(t)\mathbf{p}0 + h}(t)T\dot{\mathbf{p}0 + h}(t)\mathbf{p1 + h_1$}(t)T\dot{\mathbf{p}

其中Hermite基函数为：

h00(t)=2t3−3t2+1
h10(t)=t3−2t2+t
h01(t)=−2t3+3t2
h11(t)=t3−t2

边界条件：𝐩0 为当前末端位置，𝐩˙0 为当前末端速度，𝐩1 为拦截点位置，𝐩˙1=0（抓取时刻相对速度最小化）。

### 3.5.4 MPC代价函数

完整MPC优化问题的代价函数为：

$J = \sum_{k=0}^{N_c-1} \left[ w_{pos} \cdot c_k \cdot |\mathbf{p}{tip}^k - \mathbf{p}^k) \right]$}^k|^2 + w_{\tau} \cdot |\Delta\mathbf{q}^k|^2 + w_{sing} \cdot P_{sing}(\mathbf{q

其中：
- $\mathbf{p}{tip}^k = FK(\mathbf{q}^k)为第k步的末端位置（通过正运动学计算）−\mathbf{p}^k为预测的目标位置−c_k为预测置信度，对位置误差加权−\Delta\mathbf{q}^k为关节角度增量−P_{sing}$ 为奇异位形惩罚项

奇异位形惩罚定义为：

Psing(𝐪)=1σmin(𝐉)+10−6if σmin(𝐉)<0.01 0otherwise

其中 σmin(𝐉) 为雅可比矩阵的最小奇异值。

### 3.5.5 优化求解

采用L-BFGS-B算法求解上述约束优化问题。决策变量为控制时域内的关节角度增量序列 Δ𝐪0,Δ𝐪1,…,Δ𝐪Nc−1，共 Nc×6=60 个变量。

约束条件为关节速度限制：

|Δqik|≤q˙max·Δt=3.14×0.01=0.0314 rad

采用热启动策略：将上一步的最优解左移一步作为当前步的初始猜测，加速收敛。最大迭代次数设为50次，函数容差 10−6。

## 3.6 抓取策略模块

### 3.6.1 末端执行器选择

系统模拟三种末端执行器，根据碎片特性自动选择最优方案：

| 执行器类型 | 最大抓取尺寸 | 闭合时间 | 保持力 | 速度容差 | 适用目标 |
| --- | --- | --- | --- | --- | --- |
| 三指夹爪 | 1.5 m | 0.3 s | 50 N | 0.3 m/s | 小型碎片 |
| 柔性包络爪 | 3.0 m | 0.5 s | 30 N | 0.5 m/s | 废弃卫星、不规则体 |
| 电磁吸附器 | 5.0 m | 0.1 s | 100 N | 0.8 m/s | 火箭残骸（金属） |

选择逻辑基于碎片形状与执行器的优先级映射：

（1）废弃卫星 → 柔性包络爪 > 电磁吸附器 > 三指夹爪；（2）火箭残骸 → 电磁吸附器 > 柔性包络爪；（3）不规则碎片 → 三指夹爪 > 柔性包络爪。

系统按优先级顺序检查执行器是否可用且尺寸匹配，选择第一个满足条件的执行器。

### 3.6.2 抓取时机决策

抓取时机的判断综合考虑距离、相对速度和置信度三个条件：

Grasp=Trueif d<dth∧vrel<vtol∧c>0.3 Falseotherwise

其中：
- dth=0.15 m 为距离阈值
- vtol 为所选执行器的速度容差
- c 为当前观测置信度
- $v_{rel} = |\dot{\mathbf{p}}{tip} - \dot{\mathbf{p}}|$ 为末端与碎片的相对速度

当距离在 dth2dth 范围内时，系统进入"接近"状态，开始评估抓取条件但不执行。

### 3.6.3 接近方向计算

为避免正面碰撞，系统计算最优接近方向。设碎片运动方向为 𝐯^，碎片到机械臂方向为 𝐝^，则：

𝐬=𝐯^×𝐝^

若 |𝐬|>0.1（非共线），接近方向为追赶与侧面的加权组合：

𝐚^=normalize(0.7𝐯^+0.3𝐬^)

否则从碎片运动方向的正后方追赶：𝐚^=𝐯^。

这种策略使机械臂从碎片运动方向的侧后方接近，减小抓取时刻的相对速度，提高抓取成功率。

# 4 Experimental Setup

## 4.1 Goal and Script Version

This revision uses the engineering scripts in the folder that contains:

（1）orbit_simulator_experiment.py (batch runs + CSV export)；（2）orbit_simulator.py (single baseline run + video recording)；（3）run_experiments_and_record.ps1 (one-click pipeline)。

Experiment date: 2026-03-03.
Result CSV: experiment_results/orbit_experiment_results_20260303.csv.

## 4.2 Configuration

Batch configuration:

（1）Episodes: 6；（2）Simulated time per episode: 600 s；（3）Simulation speedup: 100x；（4）Random seeds: 20260303 to 20260308；（5）Autopilot: enabled。

Video recording configuration:

（1）Baseline video:
orbit_simulator.py --sim-seconds 420 --sim-speedup 120 --autopilot --record；（2）Experiment video:
orbit_simulator_experiment.py --sim-seconds 420 --sim-speedup 120 --record。

## 4.3 Metrics

Raw fields used in this paper:

（1）catches；（2）destroyed；（3）delta_v_total；（4）sim_time；（5）spawned_debris_total。

Derived metrics:

ηi=catchesispawned_debris_totali

λi=catchesisim_timei

Destroy rate:

rdestroy=idestroyediN

## 4.4 Statistics Reported

This paper reports:

（1）mean；（2）standard deviation (std)；（3）minimum (min)；（4）maximum (max)。

# 5 Results and Analysis

## 5.1 Overall Performance

Statistics for 6 episodes (N=6) are shown in Table 5-1.

| Metric | Mean | Std | Min | Max |
| --- | --- | --- | --- | --- |
| catches | 2.167 | 0.983 | 1 | 4 |
| delta_v_total | 9945.333 | 1004.144 | 8685 | 11305 |
| sim_time (s) | 601.750 | 0.493 | 601.1 | 602.3 |
| spawned_debris_total | 8.333 | 2.582 | 5 | 10 |
| spawned_meteors_total | 0.000 | 0.000 | 0 | 0 |

Destroy rate:

rdestroy=0/6=0.000

## 5.2 Capture Efficiency

Per-episode efficiency is defined as:

ηi=catchesispawned_debris_totali

Observed values:

（1）mean efficiency: 0.300；（2）min efficiency: 0.200；（3）max efficiency: 0.800。

[Image: Fig 5-1 Capture Count by Episode] reports/exp20260303_catches.png
[Image: Fig 5-2 Capture Efficiency by Episode] reports/exp20260303_efficiency.png

## 5.3 Time and Control Cost

Episode duration is stable:

sim_time=601.750±0.493 s

Capture-frequency range:

λi=catchesisim_timei∈[0.00166, 0.00665] s−1

Equivalent time per capture is about 150.4 s to 601.1 s.

Control-cost statistics (delta_v_total): 9945.333 +/- 1004.144.

Fig 5-3 Delta-v Total by Episode

## 5.4 Stability-Return Coupling

This dataset does not include explicit speed/angular-speed bins, so we report a stability-return-cost view:

（1）Stability: destroy rate = 0.000 (no destroyed episodes).；（2）Return: mean captures = 2.167.；（3）Cost: mean delta_v_total = 9945.333.。

Return per cost is defined as:

ρ=catchesdelta_v_total

From means:

ρmean≈2.18×10−4

## 5.5 Variability

Coefficient of variation (CV):

CVΔv=1004.1449945.333=0.101

CVcatches=0.9832.167=0.454

Return-side variability is much higher than cost-side variability.

## 5.6 Per-Episode Results

| Episode | Seed | Catches | Destroyed | Sim Time (s) | Delta-v |
| --- | --- | --- | --- | --- | --- |
| 1 | 20260303 | 4 | 0 | 601.6 | 11305 |
| 2 | 20260304 | 2 | 0 | 602.2 | 9615 |
| 3 | 20260305 | 2 | 0 | 602.3 | 10195 |
| 4 | 20260306 | 2 | 0 | 601.1 | 8685 |
| 5 | 20260307 | 2 | 0 | 602.0 | 10060 |
| 6 | 20260308 | 1 | 0 | 601.3 | 9812 |

The full run is reproducible via run_experiments_and_record.ps1.

# 6 Conclusion and Future Work

## 6.1 Summary

This revision delivers an engineering-complete experiment loop:

（1）executable scripts,；（2）quantitative CSV outputs,；（3）and reproducible videos.。

## 6.2 Main Findings

From the 2026-03-03 batch run (6 episodes):

（1）Destroy rate is 0.000.；（2）Mean captures is 2.167 (range: 1 to 4).；（3）Mean delta_v_total is 9945.333 (std: 1004.144).；（4）Return CV (0.454) is higher than cost CV (0.101), indicating stronger sensitivity on outcome side.。

## 6.3 Next Steps

（1）Increase sample size to 30/60/100 episodes and report confidence intervals.；（2）Record explicit speed/angular-speed/material fields for response-surface analysis.；（3）Add multi-policy comparisons (physics/lstm/reactive) under the same evaluation protocol.；（4）Automate report generation from CSV to figures/tables.。

# 参考文献

[1] Bonnal C, Ruault J M, Desjean M C. Active debris removal: Recent progress and current trends[J]. Acta Astronautica, 2013, 85: 51-60.

[2] Shan M, Guo J, Gill E. Review and comparison of active space debris capturing and removal methods[J]. Progress in Aerospace Sciences, 2016, 80: 18-32.

[3] Flores-Abad A, Ma O, Pham K, et al. A review of space robotics technologies for on-orbit servicing[J]. Progress in Aerospace Sciences, 2014, 65: 1-26.

[4] Hochreiter S, Schmidhuber J. Long short-term memory[J]. Neural Computation, 1997, 9(8): 1735-1780.

[5] Camacho E F, Bordons C. Model predictive control[M]. Springer, 2007.

[6] Yoshikawa T. Manipulability of robotic mechanisms[J]. The International Journal of Robotics Research, 1985, 4(2): 3-9.

[7] Wampler C W. Manipulator inverse kinematic solutions based on vector formulations and damped least-squares methods[J]. IEEE Transactions on Systems, Man, and Cybernetics, 1986, 16(1): 93-101.

[8] Siciliano B, Khatib O. Springer handbook of robotics[M]. Springer, 2016.

[9] Botta E M, Sharf I, Misra A K. Contact dynamics modeling and simulation of tether nets for space-debris capture[J]. Journal of Guidance, Control, and Dynamics, 2017, 40(1): 110-123.

[10] Aghili F. A prediction and motion-planning scheme for visually guided robotic capturing of free-floating tumbling objects with uncertain dynamics[J]. IEEE Transactions on Robotics, 2012, 28(3): 634-649.

# Appendix A: 2026-03-03 Reproduction Package

## A.1 Commands

Batch run:
python orbit_simulator_experiment.py --experiment --episodes 6 --sim-seconds 600 --sim-speedup 100 --seed 20260303 --output experiment_results/orbit_experiment_results_20260303.csv

Baseline video:
python orbit_simulator.py --sim-seconds 420 --sim-speedup 120 --autopilot --record --video-fps 30 --video-path videos/baseline_orbit_simulator_20260303.mp4

Experiment video:
python orbit_simulator_experiment.py --sim-seconds 420 --sim-speedup 120 --record --video-fps 30 --video-path videos/experiment_orbit_simulator_20260303.mp4

## A.2 Artifacts

（1）experiment_results/orbit_experiment_results_20260303.csv；（2）videos/baseline_orbit_simulator_20260303.mp4；（3）videos/experiment_orbit_simulator_20260303.mp4。

## A.3 Key Stats

（1）Episodes: 6；（2）Mean captures: 2.167；（3）Destroy rate: 0.000；（4）Mean delta-v total: 9945.333。

## A.4 Note

This appendix provides a direct reproducibility chain (commands -> data -> videos), consistent with Chapter 5.